# ESP32 IoT Vehicle Pollution Regulation Code

#include <WiFi.h>
#include <WiFiClientSecure.h>

/* WiFi Credentials */
const char* ssid = "YOUR_WIFI_NAME";
const char* password = "YOUR_WIFI_PASSWORD";

/* Telegram Bot Details */
String botToken = "YOUR_BOT_TOKEN";
String chatID = "YOUR_CHAT_ID";

/* Pin Definitions */
#define GAS_SENSOR_PIN 34
#define KM_SENSOR_PIN 27
#define LED_PIN 2

/* Variables */
volatile int pulseCount = 0;
float distance_km = 0;
float gas_ppm = 0;
float emission_gpkm = 0;

float EMISSION_LIMIT = 1.0; // BS-IV CO limit (g/km)

/* KM Sensor Interrupt */
void IRAM_ATTR countPulse() {
  pulseCount++;
}

/* WiFi Connection */
void connectWiFi() {
  WiFi.begin(ssid, password);
  while (WiFi.status() != WL_CONNECTED) {
    delay(500);
  }
}

/* Telegram Message Function */
void sendTelegram(String message) {
  WiFiClientSecure client;
  client.setInsecure();

  String url = "https://api.telegram.org/bot" + botToken +
               "/sendMessage?chat_id=" + chatID +
               "&text=" + message;

  client.connect("api.telegram.org", 443);
  client.print(String("GET ") + url + " HTTP/1.1\r\n" +
               "Host: api.telegram.org\r\n" +
               "Connection: close\r\n\r\n");
}

void setup() {
  Serial.begin(115200);

  pinMode(GAS_SENSOR_PIN, INPUT);
  pinMode(KM_SENSOR_PIN, INPUT_PULLUP);
  pinMode(LED_PIN, OUTPUT);

  attachInterrupt(digitalPinToInterrupt(KM_SENSOR_PIN), countPulse, RISING);

  connectWiFi();
}

void loop() {

  pulseCount = 0;
  delay(5000);   // Measurement interval

  /* Distance Calculation */
  distance_km = pulseCount * 0.002; // Calibration factor

  /* Gas Sensor Reading */
  int gasValue = analogRead(GAS_SENSOR_PIN);
  gas_ppm = map(gasValue, 0, 4095, 0, 1000);

  if (distance_km > 0) {
    emission_gpkm = gas_ppm / distance_km;
  }

  String status;
  if (emission_gpkm <= EMISSION_LIMIT) {
    digitalWrite(LED_PIN, HIGH);
    status = "VEHICLE FIT";
  } else {
    digitalWrite(LED_PIN, LOW);
    status = "VEHICLE NOT FIT";
  }

  String message =
    "Vehicle Pollution Report\n"
    "Gas (ppm): " + String(gas_ppm) +
    "\nDistance (km): " + String(distance_km) +
    "\nEmission (g/km): " + String(emission_gpkm) +
    "\nStatus: " + status;

  sendTelegram(message);

  delay(15000);
}